18V, 2.5A Synchronous Step-Down

DC/DC Converter

| Description The FR9204FH is a synchronous step-down DC/DC converter with fast constant on time (FCOT) mode control. The device provides 4.5V to 18V input voltage range and 2.5A continuous load current capability. It is constant on time pulse width modulation (PWM) controller that supports FCOT mode control. Operation frequency depends on Input and output voltage condition. The FR9204FH fault protection includes cycle-by-cycle current limit, short circuit protection, UVLO and thermal shutdown. The soft-start function prevents inrush current at turn-on. The FR9204FH use fast constant on time control that provides fast transient response, the noise immunity and all kinds of very low ESR output capacitor for ensuring performance stabilization. The FR9204FH is offered in TSOT-23-6 and SOT-563-6 packages, which provides good thermal conductance. | Features - Low RDS(ON) Integrated Power MOSFET (80mΩ/40mΩ) - Wide Input Voltage Range: 4.5V to 18V - Output Voltage Range: 0.768V to 8V(FR9204FHS9) - Output Voltage Range: 0.8V to 8V(FR9204FHSD) - 2.5A Output Current - FCOT Mode Enables Fast Transient Response - Pseudo 1.2MHz Frequency - Input Under Voltage Lockout - Internal 1.5ms Soft-Start - Cycle-by-Cycle Current Limit - Hiccup Short Circuit Protection - Over Temperature Protection with Auto Recovery - TSOT-23-6 and SOT-563-6 Packages Applications - STB (Set-Top-Box) - LCD Display, TV - Distributed Power System - Networking, XDSL Modem |
| --- | --- |

| Pin Assignments S9 Package: TSOT-23-6. SD Package: SOT-563-6. Figure 1. Pin Assignments of FR9204FH. | Ordering Information FR9204FH□ Package Type S9: TSOT-23-6 SD: SOT-563-6. Marking Part Number; Product Code. FR9204FHS9; CH4. FR9204FHSD; At. |
| --- | --- |

Typical Application Circuit

Figure 2. FR9204FHS9 Application Circuit

VIN=12V, the recommended BOM list is as below.

| VOUT | C1 | R1 | R2 | C4 | L1 | C2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1.05V | 22μF MLCC | 3.74kΩ | 10kΩ | 68pF~220pF | 0.68μH | 22μF MLCC x2 |
| 1.2V | 22μF MLCC | 5.76kΩ | 10kΩ | 68pF~220pF | 0.68μH | 22μF MLCC x2 |
| 1.8V | 22μF MLCC | 13.7kΩ | 10kΩ | 68pF~220pF | 1μH | 22μF MLCC x2 |
| 2.5V | 22μF MLCC | 22.6kΩ | 10kΩ | 68pF~220pF | 1μH | 22μF MLCC x2 |
| 3.3V | 22μF MLCC | 33.2kΩ | 10kΩ | 68pF~220pF | 1.5μH | 22μF MLCC x2 |
| 5V | 22μF MLCC | 54.9kΩ | 10kΩ | 68pF~220pF | 1.5μH | 22μF MLCC x2 |

Table 1. FR9204FHS9 Recommended Component Values

Typical Application Circuit (Continued)

Figure 3. FR9204FHSD Application Circuit

VIN=12V, the recommended BOM list is as below.

| VOUT | C1 | R1 | R2 | C4 | L1 | C2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1.05V | 22μF MLCC | 4.87kΩ | 16.2kΩ | 68pF~220pF | 0.68μH | 22μF MLCC x2 |
| 1.2V | 22μF MLCC | 4.87kΩ | 10kΩ | 68pF~220pF | 0.68μH | 22μF MLCC x2 |
| 1.8V | 22μF MLCC | 4.87kΩ | 3.92kΩ | 68pF~220pF | 1μH | 22μF MLCC x2 |
| 2.5V | 22μF MLCC | 4.87kΩ | 2.32kΩ | 68pF~220pF | 1μH | 22μF MLCC x2 |
| 3.3V | 22μF MLCC | 30.1kΩ | 9.76kΩ | 68pF~220pF | 1.5μH | 22μF MLCC x2 |
| 5V | 22μF MLCC | 30.1kΩ | 5.76kΩ | 68pF~220pF | 1.5μH | 22μF MLCC x2 |

Table 2. FR9204FHSD Recommended Component Values

Functional Pin Description

| Pin Name | Pin No. TSOT-23-6 | Pin No. SOT-563-6 | Pin Function |
| --- | --- | --- | --- |
| GND | 1 | 3 | Ground pin. |
| LX | 2 | 2 | Power switching node. Connect an external inductor to this switching node. |
| VIN | 3 | 1 | Power supply input pin. Placed input capacitors as close as possible from VIN to GND to avoid noise influence. |
| FB | 4 | 6 | Voltage feedback input pin. Connect FB and VOUT with a resistive voltage divider. This IC senses feedback voltage via FB and regulates it at 0.768V (FR9204FHS9) or 0.8V (FR9204FHSD). |
| SHDN | 5 | 5 | Enable input pin. Pull high to turn on IC, and pull low to turn off IC. Connect VIN with a 100kΩ resistor for self-startup. |
| BST | 6 | 4 | High side gate drive boost pin. A capacitance between 100nF~1μF must be connected from this pin to LX. It can boost the gate drive to fully turn on the internal high side NMOS. |

Block Diagram

Figure 4. Block Diagram of FR9204FH

Absolute Maximum Ratings (Note 1)

| ● Supply Voltage VIN ------------------------------------------------------------------------------------------- |  | -0.3V to +20V |  |
| --- | --- | --- | --- |
| ● Enable Voltage VSHDN ------------------------------------------------------------------------------------- |  | -0.3V to +20V |  |
| ● LX Voltage VLX ------------------------------------------------------------------------------------------------ |  | -0.3V to (VIN +0.3V) |  |
| ● Dynamic LX Voltage in 15ns Duration------------------------------------------------------------------- |  | -5V to VIN +5V |  |
| ● BST Pin Voltage VBST --------------------------------------------------------------------------------------- |  | -0.3V to (VLX +6.5V) |  |
| ● All Other Pins Voltage -------------------------------------------------------------------------------------- |  | -0.3V to +6V |  |
| ● Maximum Junction Temperature (TJ) ------------------------------------------------------------------- |  | +150°C |  |
| ● Storage Temperature (TS) --------------------------------------------------------------------------------- |  | -65°C to +150°C |  |
| ● Lead Temperature (Soldering, 10sec.) ----------------------------------------------------------------- |  | +260°C |  |
| ● Package Thermal Resistance, (θJA) (Note 2) |  |  |  |
|  | TSOT-23-6 ----------------------------------------------------------------------------------------- | 85°C/W |
|  | SOT-563-6 ----------------------------------------------------------------------------------------- | 130°C/W |
| ● Package Thermal Resistance, (θJC) |  |  |  |
|  | TSOT-23-6 ----------------------------------------------------------------------------------------- | 20°C/W |
|  | SOT-563-6 ----------------------------------------------------------------------------------------- | 60°C/W |

Note 1: Stresses beyond this listed under “Absolute Maximum Ratings" may cause permanent damage to the device.

Note 2: θJA is measured at 25°C ambient with the component mounted on a high effective thermal conductivity 4-layer board of JEDEC-51-7. The thermal resistance greatly varies with layout, copper thickness, number of layers and PCB size.

Recommended Operating Conditions

| ● Supply Voltage VIN ------------------------------------------------------------------------------------------ | +4.5V to +18V |
| --- | --- |
| ● Operating Ambient Temperature Range -------------------------------------------------------------- | -40°C to +85°C |
| ● Operating Junction Temperature Range -------------------------------------------------------------- | -40°C to +125°C |

Electrical Characteristics

(VIN=12V, TA=25°C, unless otherwise specified.)

| Parameter | Symbol | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VIN Quiescent Current | IDDQ | VSHDN=2V, VFB=1V |  | 0.3 |  | mA |
| VIN Shutdown Supply Current | ISD | VSHDN=0V |  | 0.1 | 1 | μA |
| Feedback Threshold Voltage (FR9204FHS9) | VFB | 4.5V≦VIN≦18V | 0.757 | 0.768 | 0.779 | V |
| Feedback Threshold Voltage (FR9204FHSD) | VFB | 4.5V≦VIN≦18V | 0.788 | 0.8 | 0.812 | V |
| Feedback Input Current | IFB | VFB=1V |  | 0.01 | 0.1 | μA |
| High-Side MOSFET RDS(ON) | RDS(ON) |  |  | 80 |  | mΩ |
| Low-Side MOSFET RDS(ON) | RDS(ON) |  |  | 40 |  | mΩ |
| Valley Current Limit (Note 3) | ILIMIT |  | 3.3 | 4 | 5.3 | A |
| On Time (Note 3) | TON | VIN=12V, VOUT=1.05V |  | 100 |  | ns |
| Minimum Off Time | TOFF(MIN) | VFB=0.4V |  | 150 |  | ns |
| Input Supply Voltage UVLO Threshold | VUVLO(Vth) | VIN Rising | 3.7 | 4 | 4.3 | V |
| UVLO Threshold Hysteresis | VUVLO(HYS) |  |  | 0.25 |  | V |
| Internal Soft-Start Period (Note 3) | TSS |  |  | 1.5 |  | ms |
| SHDN Input Low Voltage | VSHDN(L) |  |  |  | 0.5 | V |
| SHDN Input High Voltage | VSHDN(H) |  | 1.5 |  |  | V |
| SHDN Input Current | ISHDN | VSHDN=2V |  | 2 |  | μA |
| Thermal Shutdown Threshold (Note 3) | TSD |  |  | 160 |  | °C |
| Thermal Shutdown Hysteresis (Note 3) | THYS |  |  | 30 |  | °C |

Note 3: Not production tested.

Typical Performance Curves

| VOUT=1.05V | VOUT=3.3V |
| --- | --- |
| Figure 5. Efficiency vs. Load Current | Figure 6. Efficiency vs. Load Current |
| VOUT=5.0V | VIN=12V, VOUT=3.3V, IOUT=0A~2.5A |
| Figure 7. Efficiency vs. Load Current | Figure 8. Switch Frequency vs. Load Current |
| VSHDN=2V, VFB=1V | VIN=12V |
| Figure 9. Quiescent Current vs. Ambient Temperature | Figure 10. Feedback Voltage vs. Ambient Temperature |

Typical Performance Curves (Continued)

VIN=12V, VOUT=3.3V, C1=22μF, C2=22μFx2, C4=82pF, L1=4.7μH, TA=+25°C, unless otherwise noted.

| VLX 5V/div. IOUT=0A IL 1A/div. VOUT 10mV/div. 1μs/div. | IOUT=2.5A IL 2A/div. VOUT 10mV/div. VLX 5V/div. 1μs/div. |
| --- | --- |
| Figure 11. Steady State Waveform | Figure 12. Steady State Waveform |
| IOUT=0A IL 1A/div. VOUT 2V/div. VIN 5V/div. VLX 5V/div. 2ms/div. | IOUT=2.5A IL 2A/div. VOUT 2V/div. VLX 5V/div. VIN 5V/div. 2ms/div. |
| Figure 13. Power On Through VIN Waveform | Figure 14. Power On Through VIN Waveform |
| IOUT=0A VIN 5V/div. VLX 5V/div. VOUT 2V/div. IL 1A/div. 40ms/div. | IOUT=2.5A VIN 5V/div. VLX 5V/div. VOUT 2V/div. IL 2A/div. 40ms/div. |
| Figure 15. Power Off Through VIN Waveform | Figure 16. Power Off Through VIN Waveform |

Typical Performance Curves (Continued)

VIN=12V, VOUT=3.3V, C1=22μFx1, C2=22μFx2, C4=82pFx1, L1=4.7μH, TA=+25°C, unless otherwise noted.

| IOUT=0A IL 1A/div. VOUT 2V/div. VLX 5V/div. VSHDN 5V/div. 2ms/div. | IOUT=2.5A IL 2A/div. VOUT 2V/div. VLX 5V/div. VSHDN 5V/div. 2ms/div. |
| --- | --- |
| Figure 17. Power On Through SHDN Waveform | Figure 18. Power On Through SHDN Waveform |
| IOUT=0A IL 1A/div. VOUT 2V/div. VLX 5V/div. VSHDN 5V/div. 40ms/div. | IOUT=2.5A IL 2A/div. VOUT 2V/div. VLX 5V/div. VSHDN 5V/div. 40ms/div. |
| Figure 19. Power Off Through SHDN Waveform | Figure 20. Power Off Through SHDN Waveform |
| IOUT=1.5A to 2.5A VOUT 100mV/div. IL 1A/div. 40μs/div. |  |
| Figure 21. Load Transient Waveform |  |

Function Description

| The FR9204FH is a synchronous step-down DC/DC converter with fast constant on time (FCOT) mode control. It has integrated high-side (80mΩ, typ) and low-side (40mΩ, typ) power switches, and provides 2.5A continuous load current. It regulates input voltage from 4.5V to 18V, and down to an output voltage as low as 0.768V (FR9204FHS9) or 0.8V (FR9204FHSD). Using FCOT control scheme provides fast transient response, which can minimize the component size without additional external compensation network. Enable The FR9204FH SHDN pin provides digital control to turn on/turn off the regulator. When the voltage of SHDN exceeds the threshold voltage, the regulator starts the soft start function. If the SHDN pin voltage is below than the shutdown threshold voltage, the regulator will turn into the shutdown mode and the shutdown current will be smaller than 1μA. For auto start-up operation, connect SHDN to VIN through a 100kΩ resistor. Soft Start The FR9204FH employs internal soft start function to reduce input inrush current during start up. The typical value of internal soft start time is 1.5ms. Input Under Voltage Lockout When the FR9204FH is power on, the internal circuits are held inactive until VIN voltage exceeds the input UVLO threshold voltage. And the regulator will be disabled when VIN is below the input UVLO threshold voltage. The hysteretic of the UVLO comparator is 250mV (typ). | Over Current Protection The FR9204FH over current protection function is implemented using cycle-by-cycle current limit architecture. The inductor current is monitored by Low-side MOSFET. When the load current increases, the inductor current also increases. When the valley inductor current reaches the current limit threshold, the output voltage starts to drop. When the over current condition is removed, the output voltage returns to the regulated value. Short Circuit Protection The FR9204FH provides short circuit protection function to prevent the device damage from short condition. When the short condition occurs and the feedback voltage drops lower than 0.33V, the oscillator frequency will be reduced naturally and hiccup mode will be triggered to prevent the inductor current increasing beyond the current limit. Once the short condition is removed, the frequency will return to normal. Over Temperature Protection The FR9204FH incorporates an over temperature protection circuit to protect itself from overheating. When the junction temperature exceeds the thermal shutdown threshold temperature, the regulator will be shutdown. And the hysteretic of the over temperature protection is 30°C (typ). |
| --- | --- |

Application Information

| Output Voltage Setting The output voltage VOUT is set using a resistive divider from the output to FB. The FB pin regulated voltage is 0.768V or 0.8V. Thus the output voltage equation is: VOUT=0.768V×1+R1R2...(FR9204FHS9) VOUT=0.8V×1+R1R2…..(FR9204FHSD) Place resistors R1 and R2 close to FB pin to prevent stray pickup. Input Capacitor Selection The use of the input capacitor is filtering the input voltage ripple and the MOSFETS switching spike voltage. Because the input current to the step-down converter is discontinuous, the input capacitor is required to supply the current to the converter to keep the DC input voltage. The capacitor voltage rating should be 1.25 to 1.5 times greater than the maximum input voltage. The input capacitor ripple current RMS value is calculated as: ICIN(RMS)=IOUT×D×1-D D=VOUTVIN Where D is the duty cycle of the power MOSFET. This function reaches the maximum value at D=0.5 and the equivalent RMS current is equal to IOUT/2. The following diagram is the graphical representation of above equation. 2A 1A 2.5A A low ESR capacitor is required to keep the noise minimum. Ceramic capacitors are better, but tantalum or low ESR electrolytic capacitors may also suffice. When using tantalum or electrolytic capacitors, a 0.1μF ceramic capacitor should be placed as close to the IC as possible. | Output Capacitor Selection The output capacitor is used to keep the DC output voltage and supply the load transient current. When operating in constant current mode, the output ripple is determined by four components: VRIPPLEt=VRIPPLECt+VRIPPLEESRt +VRIPPLE(ESL)t+VNOISEt The following figures show the form of the ripple contributions. VRIPPLE(ESR)(t) (t) + VRIPPLE(ESL) (t) (t) + VRIPPLE(C) (t) (t) + VNOISE (t) = VRIPPLE(t) (t) |
| --- | --- |

Application Information (Continued)

| VRIPPLE(ESR)=VOUTFOSC×L×1-VOUTVIN×ESR VRIPPLE(ESL)=ESLL×VIN VRIPPLE(C)=VOUT8×FOSC2×L×COUT×1-VOUTVIN Where FOSC is the switching frequency, L is the inductance value, VIN is the input voltage, ESR is the equivalent series resistance value of the output capacitor, ESL is the equivalent series inductance value of the output capacitor and the COUT is the output capacitor. Low ESR capacitors are preferred to use. Ceramic, tantalum or low ESR electrolytic capacitors can be used depending on the output ripple requirement. When using the ceramic capacitors, the ESL component is usually negligible. Probe Ground It is important to use the proper method to eliminate high frequency noise when measuring the output ripple. The figure shows how to locate the probe across the capacitor when measuring output ripple. Removing the scope probe plastic jacket in order to expose the ground at the tip of the probe. It gives a very short connection from the probe ground to the capacitor and eliminating noise. Ceramic Capacitor VOUT GND Inductor Selection The output inductor is used for storing energy and filtering output ripple current. But the trade-off condition often happens between maximum energy storage and the physical size of the inductor. The first consideration for selecting the output inductor is to make sure that the inductance is large enough to keep the converter in the continuous current mode. | That will lower ripple current and result in lower output ripple voltage. The ΔIL is inductor peak-to-peak ripple current: ∆IL=VOUTFOSC×L×1-VOUTVIN A good compromise value between size and efficiency is to set the peak-to-peak inductor ripple current ΔIL equal to 30% of the maximum load current. But setting the peak-to-peak inductor ripple current ΔIL between 20%~50% of the maximum load current is also acceptable. Then the inductance can be calculated with the following equation: ∆IL=0.3×IOUT(MAX) L=VIN-VOUT×VOUTVIN×FOSC×∆IL To guarantee the required output current, the inductor needs a saturation current rating and a thermal rating that exceeds IL (peak current). These are minimum requirements. To maintain control of inductor current in overload and short circuit conditions, some applications may desire current ratings up to the current limit value. External Diode Selection For 5V input applications, it is recommended to add an external boost diode. This helps improving the efficiency. The boost diode can be a low cost one such as 1N4148. |
| --- | --- |

Outline Information

| TSOT-23-6 Package (Unit: mm) |  |  |
| --- | --- | --- |
|  |  | SYMBOLS UNIT; DIMENSION IN MILLIMETER. MIN; MAX. A; 0.70; 0.95. A1; 0.00; 0.10. A2; 0.70; 0.85. B; 0.30; 0.50. D; 2.80; 3.00. E; 2.60; 3.00. E1; 1.50; 1.70. e; 0.90; 1.00. e1; 1.80; 2.00. C; 0.08; 0.20. L; 0.30; 0.60. |

Note4: Body dimensions do not include mold flash or protrusion. Mold flash and protrusion shall not exceed 0.3mm.

Carrier Dimensions

Outline Information (Continued)

| SOT-563-6 Package (Unit: mm) |  |
| --- | --- |
|  | SYMBOLS UNIT; DIMENSION IN MILLIMETER. MIN; MAX. A; 0.525; 0.600. A1; 0.000; 0.050. B; 0.170; 0.270. D; 1.500; 1.700. E; 1.500; 1.700. E1; 1.100; 1.300. e; 0.500 BSC. L; 0.100; 0.300. L1; 0.200; 0.400. |

Note 5: Body dimensions do not include mold flash or protrusion. Mold flash and protrusion shall not exceed 0.3mm.

Carrier Dimensions

Life Support Policy

Fitipower’s products are not authorized for use as critical components in life support devices or other medical systems.